REPORT ON JOINTS, DEGREES OF FREEDOM AND GRUBLER’S FORMULA

Maxon Rodrigues 
First Year Student 
B.Tech in Computer Science and Engineering 
Manipal Institute of  Technology
Manipal, Karnataka, India
maxonrodrigues@gmail.com

Abstract—This document is a report on basic mechanical concepts like joints and links, degrees of freedom and Grubler’s Formula. This report will understand the different types of joints and their total possible degrees of freedom.

Keywords—Links, Joints, Degrees of Freedom, Grubler’s Formula

# Introduction

When we analyse different kinds of mechanical objects, an integral part about understanding becomes knowing how they move. It also requires understanding in mathematics to describe how physical components interact to transmit forces and generate controlled motions. This provides the foundation for designing machinery ranging from simple planar linkages to complex six degrees of freedom joints. So, one must understand the relationships between joints, and their degrees of freedom. Finally, this report will include Grubler’s formula and its importance in determining the degrees of freedom in two-dimensional planar mechanisms.

# Links

A kinematic link is defined as a rigid or resistant body or an assembly of them, that forms part of a mechanism, containing attachment points also called nodes, and moves relative to other parts to transmit force and motion. A link may not be perfectly rigid since a 100% rigid body doesn’t exist. However, it must be a resistant body, which transmits required forces with negligible deformation under operating conditions.  Let us now investigate the different classification of links.

## Based on Number of Nodes

Links are classified by the number of active nodes available for joint connection. Some of them are:

- Binary Links: A link possessing exactly two nodes.

- Ternary Links: A link possessing three nodes.

- Quaternary Links: A link possessing four nodes.

- Quinary Links: A link possessing five nodes.

- Polynary Link: A link possessing multiple nodes, allowing connections to multiple other links.

## Based on the Functional Role

Every kinematic link has a specific role in a mechanism to transform input motion into output motion. Some of them are:

- Fixed Link (also called Ground or Frame): It is the reference link that remains stationary relative to the frame of reference. Every mechanism must have exactly one fixed link.

- Input Link (also called Driver or Crank): The link connected directly to the power source that provides the initial driving motion.

- Connecting Link (also called a Coupler): A link connected to two other moving links that floats freely without being directly attached to the fixed ground link. It transmits motion across the mechanism.

- Output Link (also called Follower, Rocker or Slider): The link that delivers the final desired motion or work output.

## Based on the Resistance from deformation.

Links are classified by how resistant they are to deformation under stress. However, no matter their rigidity, they must be a resistant body. Some of them are:

- Rigid Link: It undergoes negligible deformation in all directions. It is used to transmit push, pull, and twist equally well. Some examples are connecting rods, pistons and crankshafts.

- Flexible Link: It deforms easily in general but resists deformation along specific loading paths. Transmits power strictly under tension. Cannot transmit push. Some examples are rubber timing belts, steel wire ropes, roller chains.

- Fluid Link: It takes the shape of its container and flows freely when unconfined. It transmits power strictly under hydrostatic compression or under pressure inside sealed systems. Some examples are hydraulic brake oil, hydraulic fluid in lifts/presses.

# Joints And Degrees of Freedom

Joints also known as kinematic pairs are the movable junction between two or more links that permits relative motion between them while constraining unneeded movement. Generally, in a three-dimensional space, an unconstrained rigid link has six degrees of freedom where three translations are along the x, y and z axes and three rotations around the same axes. Joining links together imposes mechanical constraints, reducing the allowable motion to specific input paths. The number of independent movements a joint allows are called the Allowed Degrees of freedom denoted by the letter (fi). And the number of movements it blocks are called Constrained Degrees of Freedom denoted by the letter (ci). Hence, in 3D spatial kinematics, we get:

fi+ci=6

Joints are classified as follows:

## Based on the Contact type.

- Lower Pairs: Here, the links meet along surface-to-surface contact areas. For example, a pin in a hole or a block in a slider. These distribute wear evenly and support heavy loads. Some standard lower pairs include the revolute (R), prismatic (P), helical (H), cylindrical (C), universal (U), spherical (S), and planar (E) joints.

- Higher Pairs: Here, the links meet at point or line contacts. For example, gear mesh (G) and cam-and-follower (Cam). Higher pairs typically allow more complex relative motions, combining translation and rotation simultaneously.

## Based on the number of joined links.

- Binary Joint: A joint that connects 2 links at one pivot pin. It counts as 1 joint in calculations.

- Ternary Joint: A joint that connects 3 links at one pivot pin. It counts as 2 binary joints in calculations.

- Polynary Joint: Connects n links at one pivot pin. It counts as n-1 binary joints.

Now let us go through the different types of joints and their degrees of freedom:

Joint type: Revolute

- Contact category: Lower Pair

- Spatial DOFs: 1

- Planar DOFs: 1

- Spatial constrains: 5

Joint type: Prismatic

- Contact category: Lower Pair

- Spatial DOFs: 1

- Planar DOFs: 1

- Spatial constrains: 5

Joint type: Helical/ Screw

- Contact category: Lower pair

- Spatial DOFs: 1

- Planar DOFs: N/A

- Spatial constrains: 5

Joint type: Cylindrical

- Contact category: Lower Pair

- Spatial DOFs: 2

- Planar DOFs: N/A

- Spatial constrains: 4

Joint type: Universal

- Contact category: Lower Pair

- Spatial DOFs: 2

- Planar DOFs: N/A

- Spatial constrains: 4

Joint type: Spherical

- Contact category: Lower Pair

- Spatial DOFs: 3

- Planar DOFs: N/A

- Spatial constrains: 3

Joint type: Planar

- Contact category: Lower pair

- Spatial DOFs: 1

- Planar DOFs: 1

- Spatial constrains: 5

Joint type: Roll Slide

- Contact category: Higher Pair

- Spatial DOFs: 5

- Planar DOFs: 2

- Spatial constrains: 1

Joint type: Gear Mesh

- Contact category: Higher Pair

- Spatial DOFs: 5

- Planar DOFs: 2

- Spatial constrains: 1

Joint type: Cam and Follower

- Contact category: Higher Pair

- Spatial DOFs: 5

- Planar DOFs: 2

- Spatial constrains: 1

Joint type: Pin in Slot

- Contact category: Higher Pair

- Spatial DOFs: 5

- Planar DOFs: 2

- Spatial constrains: 1

# Grublers Formula

Grubler’s formula is an algebraic formula used to calculate the degree of freedom (F), or mobility, of a planar kinematic mechanism. Mobility represents the number of independent input parameters like motor inputs or actuators that are required to uniquely define the position and configuration of all links in the mechanism.

## Mathematical Formulation

For a mechanism constrained to move in a single two-dimensional plane, Grubler’s formula is expressed as:

F=3n-1-2j1-j2

Where:

- F: Degree of freedom (mobility) of the system.

- n: Total number of links in the system (including the fixed ground/frame, where n≥3).

- j1: Total number of 1-DOF lower pairs (For Example: revolute pin joints, prismatic sliding joints).

- j2: Total number of 2-DOF higher pairs (For Example: gear tooth meshes).

## Theoretical Derivation

The physical logic underlying Grubler's formula is derived from spatial constraints in a planar space:

- Unconstrained Motion: A single free body moving in a plane possesses 3 degrees of freedom (2 translational coordinates x,y and 1 rotational angle θ). A system of n disconnected links therefore possesses 3n total DOFs.

- Ground Constraint: Fixing one link as the static ground reference frame eliminates 3 DOFs, reducing total available system motion to 3n-1.

- Joint Constraints:

  - Each 1-DOF joint (j1) permits 1 motion while locking 2 independent motions, removing 2 DOFs from the system (-2j1).

  - Each 2-DOF joint (j2) permits 2 motions while locking 1 independent motion, removing 1 DOF from the system (-j2).

## Compound Joint Adjustment

When m links (m≥3) share a single common pivot pin, the junction constitutes a compound joint. To preserve mathematical validity in Grübler's equation, the compound joint must be converted to an equivalent number of binary 1-DOF joints:

j1=m-1

## Physical Interpretation of F

| Calculated Value | System Classification | Physical Behavior |
| --- | --- | --- |
| F>0 | Kinematic Mechanism | System possesses motion. Requires F independent actuators to control uniquely. |
| F=0 | Statically Determinate Structure | Rigid frame; zero relative motion between links under load. |
| F<0 | Statically Indeterminate Structure | Over-constrained structure; contains redundant links causing internal pre-stress. |

##### Acknowledgment

I would like to thank the team of Robo Manipal to give us this opportunity to learn more about the basics of mechanisms of sensors and for guiding and helping us.

##### References

Lorem ipsum